Experiment :5.1

Fourier Series Analysis of Periodic Signals

OBJECTIVE:

- To analyze Fourier series of continuous-time periodic signals and analyze their waveform characteristics.

- To determine the Fourier coefficients and harmonic components of periodic signals.

- To generate and analyze the amplitude and phase spectra of periodic signals.

- To reconstruct periodic signals using Fourier series and verify the reconstructed waveform with the original signal.

Experimental Procedure:

1. Define the time vector and fundamental frequency.

2. Generate the half-wave rectified sinusoidal periodic signal.

3. Compute the Fourier coefficients up to N = 10.

4. Construct the Fourier-series approximation.

5. Plot the original and reconstructed signals and compare them.

Matlab coding:

clc; clear; close all; %%

5(i) Analysis of Fourier Series Representation of Continuous-Time Periodic Signals figure;

% Fourier Series Representation of a Periodic Signal

A = 1;

f0 = 1;

T = 1/f0;

t = linspace(0, 2*T, 2000);

% Half-wave rectified sinusoidal signal

x = A*sin(2*pi*f0*t);

x(x < 0) = 0;

N = 10;

a0 = (2/T)*trapz(t, x)/2;

an = zeros(1,N);

bn = zeros(1,N);

for k = 1:N

an(k) = (2/T)*trapz(t, x.*cos(2*pi*k*f0*t));

bn(k) = (2/T)*trapz(t, x.*sin(2*pi*k*f0*t));

end

xFS = a0*ones(size(t));

for k = 1:N

xFS = xFS + an(k)*cos(2*pi*k*f0*t) ... + bn(k)*sin(2*pi*k*f0*t);

end

plot(t,x,'k','LineWidth',1.2); hold on;

plot(t,xFS,'r--','LineWidth',1.2);

grid on;

xlabel('Time (s)'); ylabel('Amplitude');

title('Fourier Series Representation of a Periodic Signal');

legend('Original Signal','Fourier Series Reconstruction');

Output:

Result:

The impulse response of the discrete-time system y(n)=x(n)+0.5x(n−1) was obtained by applying a unit impulse signal as input. The resulting impulse response was h(n)={1,0.5,0,0,…}, which characterizes the behavior of the system.

EXPERIMENT:5.4

To reconstruct periodic signals using Fourier series and verify the reconstructed waveform with the original signal.

Expected Outcome:

• Reconstruct the periodic waveform using the calculated coefficients.

• Plot original and reconstructed signals together.

• Obtain the reconstruction error.

• Assess the quality of the Fourier-series approximation.

MATLAB Coding:

clc; clear; close all;

%% 5(iv) Verification of Fourier Series Reconstruction and Analysis of Signal Characteristics

figure;

% Fourier Series Reconstruction and Error Analysis

f0 = 1;

T = 1/f0;

t = linspace(0,2*T,4000);

x = max(sin(2*pi*f0*t),0);

N = 10;

a0 = (2/T)*trapz(t(1:2000),x(1:2000));

an = zeros(1,N); bn = zeros(1,N);

tt = t(1:2000); xx = x(1:2000);

for k = 1:N

an(k) = (2/T)*trapz(tt,xx.*cos(2*pi*k*f0*tt));

bn(k) = (2/T)*trapz(tt,xx.*sin(2*pi*k*f0*tt));

end

xr = a0/2*ones(size(t));

for k = 1:N

xr = xr + an(k)*cos(2*pi*k*f0*t) + bn(k)*sin(2*pi*k*f0*t);

end

err = x-xr;

subplot(2,1,1);

plot(t,x,'k',t,xr,'r--','LineWidth',1.1);

grid on; xlabel('Time (s)'); ylabel('Amplitude');

title('Original and Reconstructed Signals');

legend('Original','Reconstructed');

subplot(2,1,2);

plot(t,err,'b'); grid on;

xlabel('Time (s)'); ylabel('Error');

title('Reconstruction Error');

Output:

Result:

The discrete-time system described by the difference equation y(n)−0.5y(n−1)=x(n) was analyzed using MATLAB. The output response to a unit step input was obtained and observed to gradually approach the steady-state value of 2, confirming the stable behavior of the system.

Experiment – 6.1

(i) To verify Parseval’s relation for continuous-time periodic signals and analyze the relationship between time-domain and frequency-domain power

OBJECTIVE:

(i)To verify Parseval’s relation for continuous-time periodic signals and analyze the relationship between time-domain and frequency-domain power.

(ii) To analyze the distribution of signal power among the fundamental and harmonic components of periodic signals.

(iii) To determine and analyze the harmonic spectrum of continuous-time periodic signals.

(iv) To compare the total power obtained in the time domain with the harmonic power distribution in the frequency domain.

Experimental Procedure:

• Generate the periodic signal over one period.

• Compute time-domain average power.

• Calculate Fourier coefficients up to N = 20.

• Evaluate frequency-domain power using Parseval’s relation.

• Compare the two values and calculate absolute error.

Matlab coding:

clc; clear; close all;

%% 6(i) Verification of Parseval’s Relation for Periodic Signals

figure;

% Verification of Parseval's Relation

f0 = 1;

T = 1/f0;

t = linspace(0,T,4000);

x = max(sin(2*pi*f0*t),0);

% Time-domain average power

P_time = (1/T)*trapz(t,abs(x).^2);

% Fourier-series coefficients

N = 20;

a0 = (2/T)*trapz(t,x);

an = zeros(1,N); bn = zeros(1,N);

for k = 1:N

an(k) = (2/T)*trapz(t,x.*cos(2*pi*k*f0*t));

bn(k) = (2/T)*trapz(t,x.*sin(2*pi*k*f0*t));

end

% Frequency-domain power using Parseval's relation

P_freq = a0^2/4 + 0.5*sum(an.^2 + bn.^2);

fprintf('Time-domain power = %.6f\n',P_time);

fprintf('Frequency-domain power = %.6f\n',P_freq);

fprintf('Absolute error = %.6e\n',abs(P_time-P_freq));

bar([P_time P_freq]);

set(gca,'XTickLabel',{'Time Domain','Frequency Domain'});

ylabel('Average Power');

grid on;

title('Verification of Parseval''s Relation');

Output:

Result:

The convolution of the continuous-time signals x1(t)=e−tu(t) and x2(t)=u(t) was performed successfully using MATLAB. The output signal y(t)=1−e−t was obtained and plotted, demonstrating the convolution operation for continuous-time systems.

EXPERIMENT:(6.4)

(iv) To compare the total power obtained in the time domain with the harmonic power distribution in the frequency domain.

Expected Outcomes:

• Calculate time-domain power.

• Calculate DC and harmonic power components.

• Obtain total frequency-domain power.

• Compare time-domain and frequency-domain results.

Experimental Procedure:

• Generate the periodic signal and compute Fourier coefficients.

• Calculate harmonic magnitude for n = 1 to 15.

• Convert harmonic number to frequency.

• Plot the harmonic spectrum using a stem plot.

• Identify the significant components.

MATLAB Coding:

clc; clear; close all;

%% 6(iv) Comparison of Time-Domain Power with Frequency-Domain Harmonic Power figure;

% Time-Domain and Frequency-Domain Power Comparison

f0 = 1; T = 1/f0;

t = linspace(0,T,4000);

x = max(sin(2*pi*f0*t),0);

P_time = (1/T)*trapz(t,x.^2);

N = 20;

a0 = (2/T)*trapz(t,x);

an = zeros(1,N); bn = zeros(1,N);

for k = 1:N

an(k) = (2/T)*trapz(t,x.*cos(2*pi*k*f0*t));

bn(k) = (2/T)*trapz(t,x.*sin(2*pi*k*f0*t));

end

P_dc = a0^2/4;

P_harmonics = 0.5*sum(an.^2+bn.^2);

P_frequency = P_dc + P_harmonics;

fprintf('Time-domain power = %.6f\n',P_time);

fprintf('DC power = %.6f\n',P_dc);

fprintf('Harmonic power = %.6f\n',P_harmonics);

fprintf('Frequency-domain power = %.6f\n',P_frequency);

bar([P_time P_frequency]);

set(gca,'XTickLabel',{'Time Domain','Frequency Domain'});

ylabel('Average Power'); grid on;

title('Time-Domain vs Frequency-Domain Power');

Output:

Result:

Experiment – (7.2)

(ii) To determine the magnitude and phase spectra of continuous-time signals and analyze their significant spectral components.

Expected Outcomes :

• Generate the Gaussian-modulated cosine signal.

• Compute its Fourier transform approximation.

• Plot magnitude and phase spectra. • Identify the principal spectral peaks.

Experimental Procedure:

• Define t and the sampling interval.

• Generate x(t)=e^(−t²)cos(2π3t).

• Compute the FFT-based transform with N = 4096.

• Plot the signal, magnitude and unwrapped phase.

• Identify the principal spectral peaks.

MATLAB Coding:

clc; clear; close all;

%% 7(ii) Determination of Magnitude and Phase Spectra of Continuous-Time Signals figure;

% Magnitude and Phase Spectrum of a CT Signal

t = -2:0.001:2;

dt = t(2)-t(1);

x = exp(-t.^2).*cos(2*pi*3*t);

N = 4096;

X = fftshift(fft(x,N))*dt;

f = (-N/2:N/2-1)/(N*dt);

magX = abs(X);

phaseX = unwrap(angle(X));

subplot(3,1,1);

plot(t,x); grid on;

xlabel('Time (s)'); ylabel('Amplitude');

title('Continuous-Time Signal');

subplot(3,1,2);

plot(f,magX); xlim([-10 10]); grid on;

xlabel('Frequency (Hz)'); ylabel('|X(f)|');

title('Magnitude Spectrum');

subplot(3,1,3);

plot(f,phaseX); xlim([-10 10]); grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (rad)');

title('Phase Spectrum');

Output:

Result:

EXPERIMENT:(7.3)

(iii) Verification of Fourier Transform Properties and Frequency-Domain Characteristics

Expected Outcomes :

• Generate a shifted version of a Gaussian signal.

• Compute the Fourier transform of the shifted signal.

• Calculate the theoretical shifted transform.

• Compare magnitude and phase responses.

Experimental Procedure:

• Generate x(t)=e^(−t²).

• Create x(t−0.4) using interpolation.

• Compute the FFT of the shifted signal.

• Form the theoretical transform using the time-shift property.

• Compare computed and theoretical magnitude and phase.

MATLAB Coding :

clc; clear; close all;

%% 7(iii ) Verification of Fourier Transform Properties and Frequency-Domain

Characteristics

figure;

% Verification of Fourier Transform Properties

t = -2:0.001:2;

dt = t(2)-t(1);

x = exp(-t.^2);

x_shift = interp1(t,x,t-0.4,'linear',0);

X = fftshift(fft(x))*dt;

N = length(x);

f = (-N/2:N/2-1)/(N*dt);

% Time-shift property:

x(t-t0) <-> X(f)e^(-j2*pi*f*t0)

X_shift_theory = X.*exp(-1j*2*pi*f*0.4); X_shift = fftshift(fft(x_shift))*dt;

subplot(2,1,1);

plot(f,abs(X_shift),f,abs(X_shift_theory),'--');

xlim([-5 5]); grid on;

xlabel('Frequency (Hz)'); ylabel('Magnitude');

title('Time-Shift Property: Magnitude Comparison'); legend('Computed','Theoretical'); subplot(2,1,2);

plot(f,unwrap(angle(X_shift)),f,unwrap(angle(X_shift_theory)),'--');

xlim([-5 5]); grid on;

xlabel('Frequency (Hz)'); ylabel('Phase (rad)');

title('Time-Shift Property: Phase Comparison');

legend('Computed','Theoretical');

Output:

Result:

The Fourier Transform of the continuous-time signal x(t)=e−∣t∣ was computed using MATLAB. The magnitude spectrum was obtained using FFT and showed that most of the signal energy is concentrated around the zero-frequency component, illustrating the frequency-domain representation of the signal.

EXPERIMENT:(8.2)

(ii) Determination of Inverse Laplace Transform for Signal Reconstruction

Expected Outcomes:

• Obtain x(t) from X(s).

• Display the inverse-transform expression.

• Evaluate the reconstructed waveform numerically.

• Analyze the resulting time-domain response..

Experimental Procedure :

• Define X(s) symbolically.

• Apply ilaplace to obtain x(t).

• Evaluate x(t) over 0≤t≤5 s.

• Plot the reconstructed time-domain signal. Observe plot

• Compare the plotted response with the analytical expression.

MATLAB Coding:

clc; clear; close all;

Qualitative %% 8(ii) Determination of Inverse Laplace Transform for Signal Reconstruction

figure;

% Inverse Laplace Transform

% Requires Symbolic Math Toolbox

syms s t

X = 1/((s+2)*(s+3));

x = ilaplace(X,s,t);

disp('Inverse Laplace Transform x(t) =');

pretty(x);

tt = 0:0.001:5;

xx = double(subs(x,t,tt)); plot(tt,xx,'LineWidth',1.2);

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Signal Reconstructed using Inverse Laplace Transform');

Output:

Result:

The Discrete-Time Fourier Transform (DTFT) of the sequence x(n)={1,2,3,4} was computed using MATLAB. The magnitude spectrum was obtained and plotted, illustrating the frequency domain representation of the discrete-time signal.

EXPERIMENT:(8.3)

(iii) To determine the Region of Convergence (ROC) of Laplace transforms and analyze its significance in characterizing continuous-time signals and systems.

Expected Outcomes:

• Identify poles at s = −2 and s = −4.

• State the possible ROC cases.

• Relate each ROC to the corresponding signal sidedness.

• Interpret how ROC affects the system representation.

Experimental Procedure

• Define the pole locations.

• Plot the poles in the s-plane.

• Mark the vertical lines through the poles. Record result

• State the candidate ROC for each signal-sidedness case. • Interpret the relationship between poles and ROC.

MATLAB Coding

clc; clear; close all;

%% 8(iii ) Analysis of Region of Convergence (ROC) and Its Significance in System

Analysis

figure;

% ROC Analysis for a Rational Laplace Transform

% Requires Symbolic Math Toolbox

% X(s) = 1/((s+2)(s+4))

% Poles are at s = -2 and s = -4.

p = [-2 -4];

plot(real(p),imag(p),'rx','MarkerSize',10,'LineWidth',2);

hold on; xline(-2,'--'); xline(-4,'--');

grid on;

xlabel('Real(s)'); ylabel('Imag(s)');

title('Poles and Candidate Regions of Convergence');

legend('Poles','Pole at -2','Pole at -4');

fprintf('Poles are at s = -2 and s = -4.\n');

fprintf('For a right-sided signal, ROC: Re(s) > -2.\n');

fprintf('For a left-sided signal, ROC: Re(s) < -4.\n');

fprintf('For a two-sided signal, ROC: -4 < Re(s) < -2.\n');

Output:

Result:

The DTFT of a rectangular pulse was computed using MATLAB. The magnitude spectrum exhibited a sinc-like shape with a dominant main lobe and diminishing side lobes, demonstrating the frequency-domain characteristics of a finite-duration rectangular sequence.